Title:			RTL8139(A/B/BL/C/C+)/8130/8100/8100B Driver OEMSETUP DISK

History:			V4.00b  --   10/03/2001

Name:			RTSOEM.400b

Released By:		Vincent

Update reason from Oemdisk 390 to 400

| Driver Type | Old Ver | New Ver | Reason | Checked |
| --- | --- | --- | --- | --- |
| RSET8139 EXE | Ver501 | Ver501 | None Changed |  |
| MSLANMAN DOS | Ver323 | Ver323 | None Changed |  |
| MSLANMAN OS2 | Ver321 | Ver323 | 1. Driver will look for CardBus card Device ID=8138 2. When CardBus card is found, driver will close early receive FIFO, and early receive interrupt |  |
| NDIS2DOS | Ver323 | Ver323 | None Changed |  |
| NDIS2OS2 | Ver321 | Ver323 | 1. Driver will look for CardBus card Device ID=8138 2. When CardBus card is found, driver will close early receive FIFO, and early receive interrupt |  |
| NWCLIENT\DOS | Ver361 | Ver364 | 1. Change the wait byte number of early interrupt from 4 to 64. 2. Add NONEARLY keyword to set the 8139 to run at non-early interrupt mode. 3. In mini-PCI and clock run enable the early transmit parameter should limit to 0x0a. 4. Add a loop counter for early receive to prevent 8139 enter the “wait next packet” state. That will slow down the performance. |  |
| NWCLIENT\OS2 | Ver313 | Ver313 | None Changed |  |
| NWSERVER | Ver329 | Ver329 | None Changed |  |
| CLIENT32 | Ver329 | Ver329 | None Changed |  |
| RTSPKT | Ver321 | Ver330 | 1. Fix the bug about "SET_ADDRESS" call not function. Must enable EEM0,EEM1 to 1,1. 2. Fix the bug on "-n" command. |  |
| SCO | Ver314 | Ver314 | None Changed |  |
| WFW311 | Ver313 | Ver313 | None Changed |  |
| NT351 | Ver378 | Ver397 | 1. Remove Wakeup patterns when driver is removed (MiniportHalt). 2. Read MAC address directly from EEPROM. 3. Add 2 custom OID:OID_RTL8139_LINK_DUPLEX and OID_RTL8139_RESET_COUNTER. 4. For Cardbus, read MAC address directly from EEPROM. 5. In the INF file: a. Add "Smart Power Saving" parameter (analog power on/off) b. Add "WakeUp on Link Status Change" parameter c. Rename "WakeUp Frame" to "WakeUp on ARP/PING" d. Rename "APM Mode WakeUp" to "WakeUp using APM Mode". 6. When system gos into sleeping mode, set analog power to be OFF. 7. Rename "Realtek RTL8139(A/B/C/8130) ..." to "Realtek RTL8139 Family ...". 8. In the INF files: a. Add Sub_Vendor and Sub_System ID for WHQL . b. Add "Receive Buffer Size" parameter c. Rename "WakeUp on Link Status Change" to "WakeUp on Link Change", because the sentence length of previous parameter is to long for some Win98 PC. d. Remove "Smart Power Saving" parameter, OS will set the analog power ON/OFF state. 9. Create a new NDIS4 driver to support Phoenix PowerPanel (for notebook). 10. For CardBus, to support ALi chipset completely. 11. Auto-Correct the wrong SpeedDuplexMode parameter, 12. Fixed to prevent NIC backing to auto-negotiation mode, when NIC waked up from D1~D3 back to D0, if user define Wakeup Disable and set forced-speed mode. |  |
| W95OSR2 | Ver378 | Ver397 | Same as NT351 |  |
| WIN95A | Ver378 | Ver397 | Same as NT351 |  |
| WIN98 | Ver378 | Ver397 | Same as NT351 |  |
| WINNT | Ver378 | Ver397 | Same as NT351 |  |
| WINME | Ver378 | Ver397 | Same as NT351 |  |
| WIN2000 | Ver378 | Ver395 | Same as NT351 |  |
| WINXP |  | Ver397 | New |  |
| DMI | Ver1.01 | Ver1.10 | 1. Fix bug reported by ATI. 2. Support Win98 |  |
| Unix Ware 7 | Ver1.00 | Ver2.00 | 1. Initialize wakeup frames 2. Support both RTL8139/810x and RTL8139C+ in a single driver. |  |
| LINUX | Ver1.04 | Ver1.04 | Refer to : http://www.scyld.com/network/rtl8139.html |  |
| FREEBSD | Ver3.0 | Ver3.0 | Refer to : http://www.realtek.com.tw/cn/information/8139/freebsd.htm |  |
| Mac OS | Ver3.00 | Ver3.00 | None Changed |  |
| BROM |  | N/A | None Changed |  |
| RTOS |  | N/A | None Changed |  |
| WinDiag/Win2000 |  | Ver1.70 | None Changed |  |
| WinDiag/Win9x, ME |  | Ver1.70 | None Changed |  |
| WinDiag/WinNT4 |  | Ver1.70 | None Changed |  |
| RealMP/Win9x, ME |  | Ver1.20 | New |  |
| RTLed/Win9x, ME |  | Ver1.03 | New |  |
| EZSetup/Win2000, ME |  | Ver395 | New |  |