Title:			RTL8139(A/B/BL/C/8130) Driver OEMSETUP DISK

History:			V3.80  --   02/01/2000

Name:			RTSOEM.380

Released By:		Vincent

Update reason from Oemdisk 370 to 380

| Driver Type | Old Ver | New Ver | Reason |
| --- | --- | --- | --- |
| RSET8139 EXE | Ver500 | Ver501 | 1. Identify RTL8139C ID in TCR 2. Do cable test before loopback test |
| MSLANMAN DOS | Ver321 | Ver323 | - Driver will look for CardBus card Device ID=8138 - When CardBus card is found, driver will close early receive FIFO, and early receive interrupt |
| MSLANMAN OS2 | Ver321 | Ver321 | None Changed |
| NDIS2DOS | Ver321 | Ver323 | - Driver will look for CardBus card Device ID=8138 - When CardBus card is found, driver will close early receive FIFO, and early receive interrupt |
| NDIS2OS2 | Ver321 | Ver321 | None Changed |
| NWCLIENT\DOS | Ver311 | Ver360 | - RTL8138 CardBus card can be found. - Set default values to ERFTH, ERXTH_S100, ERXTH_S10.RTL8139 will use these default values for better performance. - When the driver finds the card is 8138 CardBus card, - 100Mhz early interrupt is set to disable. - 10Mhz early interrupt is set to disable. - Early receive is set to disable. |
| NWCLIENT\OS2 | Ver313 | Ver313 | None Changed |
| NWSERVER | Ver327 | Ver327 | None Changed |
| CLIENT32 | Ver327 | Ver327 | None Changed |
| RTSPKT | Ver318 | Ver318 | None Changed |
| SCO | Ver313 | Ver314 | 1. Solve the wrong pointer when the header of the packet wrap the boundary of the SW buffer |
| WFW311 | Ver313 | Ver313 | None Changed |
| NT351 | Ver364 | Ver373 | - Fix Link speed/duplex mode setting error. - Change StallExecution to Timer. - Remote 'StallExecution and use timer instead. - Cleanup send queue in Reset handler. - Identify chip version, when NDIS query power capability, return D2 on 8139A and 8130 - Add the WakeUp Frame Enable/Disable in netrts5.inf. - add PME_EN default(enable/disable) option. - CardBus : for 8139c, add scanning 0x10ec8139. No change to INF file. - Modify inf file for HCT 9.5 InfCheck test. |
| W95OSR2 | Ver364 | Ver373 | Same as NT351 |
| WIN95A | Ver364 | Ver373 | Same as NT351 |
| WIN98 | Ver364 | Ver373 | Same as NT351 |
| WINNT | Ver364 | Ver373 | Same as NT351 |
| WIN2000 | Ver364 | Ver373 | Same as NT351 |
| WIN98\REALMP | Ver1.00 | Ver1.00 | None Changed |
| WinDiag\Win9x | Ver1.01 | Ver1.01 | None Changed |
| WinDiag\WINNT4 | Ver1.01 | Ver1.01 | None Changed |
| WinDiag\WIN2000 |  | Ver1.20 | New |
| DMI | Ver1.01 | Ver1.01 | None Changed |
| Unix Ware 7 | Ver1.00 | Ver1.00 | None Changed |
| LINUX | Ver1.04 | Ver1.04 | None Changed Refer to : http://cesdis.gsfc.nasa.gov/linux/drivers/rtl8139.html |
| FREEBSD | Ver3.0 | Ver3.0 | None Changed |
| Mac OS |  | Ver1.00 | New. For MacOS 8.5.1 and 8.6 |
| BROM |  |  | New. |
| RTOS |  |  | New. |